Scheduled Maintenance for BV350
	km x 1000

miles
	Serv.

Man.
Ref.
	1

621
	10

6,214
	15

9,321
	20

12,427
	30

18,641
	40

24,855
	45

27,962
	50

31,068
	60

37,282
	70

43,496
	75

46,603
	80

49,710
	90

55,923
	100

62,137
	105

65,244
	110

68,351
	120

74,565
	130

80,778

	Safety Fasteners
	
	I


	I
	
	I
	I
	I
	
	I


	I
	I
	
	I
	I
	I
	
	I
	I
	I

	Coolant
 
	p.53
	I


	I
	
	I
	I
	I
	
	I


	I
	I
	
	I
	I
	I
	
	I
	I
	I

	Brake Fluid1
	p.54
	I


	I
	
	I
	I
	I
	
	I


	I
	I


	
	I
	I
	I
	
	I
	I
	I

	Electrical Sys / Battery
	p.69
	I


	I
	
	I
	I
	I
	
	I


	I
	I
	
	I
	I
	I
	
	I
	I
	I

	Brake Pads
	p.294
	I


	I
	
	I
	I
	I
	
	I


	I
	I


	
	I


	I
	I


	
	I


	I
	I



	Hub Oil
	p.47
	R
	
	
	
	
	I
	
	
	
	
	
	I
	
	
	
	
	I
	

	Center Stand Bracket
	p.287
	L
	L
	
	L
	L
	L
	
	L
	L
	L
	
	L
	L
	L
	
	L
	L
	L

	Valve Clearance
	p.52
	
	
	
	
	
	I
	
	
	
	
	
	I
	
	
	
	
	I
	

	Engine Oil

	p.50
	R
	R
	
	R
	R
	R
	
	R
	R
	R
	
	R
	R
	R
	
	R
	R
	R

	Engine Oil Filter
	p.51
	R
	R
	
	R
	R
	R
	
	R
	R
	R
	
	R
	R
	R
	
	R
	R
	R

	Spark Plug
	p.69
	
	R
	
	R
	R
	R
	
	R
	R
	R
	
	R
	R
	R
	
	R
	R
	R

	Engine Air Filter

	p.49
	
	C
	
	C
	C
	C
	
	C
	C
	C
	
	C
	C
	C
	
	C
	C
	C

	Belt Compartment Filter3
	
	
	C
	
	C
	C
	C
	
	C
	C
	C
	
	C
	C
	C
	
	C
	C
	C

	Brake Pumps
	p.54
	
	
	R
	
	R
	
	R
	
	R
	
	R
	
	R
	
	R
	
	R
	

	Roller Housing / Counter
	p.159
	
	
	
	C/I
	
	C/I
	
	
	C/I
	
	
	C/I
	
	C/I
	
	
	C/I
	

	Crankcase Breather

	p.171
	
	
	
	C
	
	C
	
	
	C
	
	
	C
	
	C
	
	
	C
	

	Drive Belt
	p.158
	
	
	
	R
	
	R
	
	
	R
	
	
	R
	
	R
	
	
	R
	

	CVT Rollers / Sliders
	p.160
	
	
	
	R
	
	R
	
	
	R
	
	
	R
	
	R
	
	
	R
	

	Throttle Control
	
	I


	I
	
	I
	I
	I
	
	I


	I
	I
	
	I
	I
	I


	
	I
	I
	I



	Suspension
	p.256
	I


	I
	
	I
	I
	I
	
	I


	I
	I


	
	I


	I
	I


	
	I


	I
	I



	Steering
	p.268
	A
	I
	
	I
	I
	I
	
	I


	I
	I


	
	I


	I
	I


	
	I


	I
	I



	Tire Pressure / Wear
	
	I


	I
	
	I
	I
	I
	
	I


	I
	I


	
	I


	I
	I


	
	I


	I
	I



	Road Test
	
	I


	I
	
	I
	I
	I
	
	I


	I
	I


	
	I


	I
	I


	
	I


	I
	I




I: CHECK AND CLEAN, ADJUST, LUBRICATE OR REPLACE IF NECESSARY. 

[image: image1.emf]C: CLEAN, R: REPLACE, L: LUBRICATE, A: ADJUST
Turn key to ignition and the service icon will flash for 5 seconds immediately after the ignition check and if there are less than 187.5 miles (300 km) until the next scheduled service.  Once the service mileage is reached, the icon will remain on until reset.  
Reset the service icon by holding down the MODE key for more than 10 seconds.  For first 5 seconds there will be no change. After 5 seconds the icon will slowly flash.  If the button is released before the full 10 seconds has passed the service icon will not reset.
� Replace every 2 years


� Regardless, check the level every time you use your vehicle


� Perform maintenance more frequently when riding in unusually wet or dusty areas


� Perform maintenance more frequently if you drive mainly at full acceleration





